🤖 Business AI Robot

Antigravity Development Specification

Implementation-ready product, architecture, hardware, AI, backend, app, security, testing and exhibition guide

# 0. Document Purpose

This document is the single implementation specification for the Business AI Robot. It is intended to be given to an AI coding/development agent ("Antigravity") so that the agent understands what the product is, why it exists, how the robot works, how the ESP32/backend/database/AI/mobile app interact, what must be implemented, what must not be implemented yet, how to structure the code, and how to test the system.

Important development rule: do not replace the architecture with a different stack unless explicitly approved. Build incrementally and keep interfaces modular.

# 1. Product Definition

## 1.1 Product Name

Business AI Robot

Working concept: A 24/7 digital business assistant/manager that combines voice interaction, business software, AI reasoning and physical IoT control.

## 1.2 Product Goal

The robot is intended to automate repetitive operational work normally handled by a small-business manager, while keeping the owner in control of important or sensitive decisions.

Check stock and inventory.

Add or reduce stock.

Create bills and invoices.

Check customer balances and history.

Record payments.

Create reminders.

Provide sales and business summaries.

Provide proactive alerts.

Control connected hardware.

Provide business recommendations.

Correct product position: "automate repetitive operational management and act as a digital manager assistant."

# 2. Core Differentiator

The product is not unique merely because it uses AI. The differentiator is the combination of natural voice interaction, business context, a business database, voice-to-business actions, voice-to-physical actions, proactive alerts, multi-step workflows, a mobile business-management app, a physical ESP32 robot, and one shared backend/database.

OWNER SPEAKS
    |
    v
VOICE INPUT
    |
    v
SPEECH-TO-TEXT
    |
    v
AI / INTENT ENGINE
    |
    v
STRUCTURED COMMAND
    |
    v
BACKEND VALIDATION + PERMISSIONS
    |
    +---------------------+
    |                     |
    v                     v
BUSINESS SERVICES      ROBOT SERVICES
    |                     |
    v                     v
DATABASE             ESP32 / HARDWARE
    |
    v
VOICE RESPONSE / APP UPDATE

# 3. Final System Architecture

INTERNET
                            |
                  +---------v---------+
                  |  CLOUD BACKEND   |
                  | FastAPI          |
                  | AI service       |
                  | Business logic   |
                  | Auth             |
                  | Automation       |
                  | Robot API        |
                  +---------+--------+
                            |
              +-------------+-------------+
              |                           |
          HTTPS / API                HTTPS / API
              |                           |
      +-------v-------+           +-------v-------+
      |   MOBILE APP  |           |   ESP32 ROBOT |
      |    Flutter    |           | Mic           |
      +---------------+           | Speaker       |
                                  | OLED          |
                                  | Buttons       |
                                  | Relay         |
                                  +---------------+

## 3.1 Shared Brain Principle

The mobile app and robot must not maintain separate business databases. Both use the same backend and business database.

Robot: "Add 20 kg sugar"
        |
        v
Backend
        |
        v
Database: Sugar stock +20 kg
        |
        +--> App automatically shows new stock

# 4. Runtime Modes

## 4.1 Development Mode (current)

Physical microphone and speaker are pending delivery. Use the PC as temporary development audio infrastructure.

PC Microphone
    |
    v
PC Voice Service
    |
    v
Gemini / Backend
    |
    v
ESP32
    |
    v
Relay
    |
    v
PC Speaker / TTS

## 4.2 Hardware V1

INMP441 -> ESP32
ESP32 -> MAX98357A -> Speaker
ESP32 -> OLED
ESP32 -> Buttons
ESP32 -> Relay

## 4.3 Exhibition / Production Mode

ROBOT
  |
  | Wi-Fi
  v
INTERNET
  |
  v
CLOUD BACKEND
  |
  +--> Gemini
  +--> Database
  +--> Business services
  |
  +<-> MOBILE APP

PC is NOT required.

# 5. Hardware Specification

## 5.1 Current Hardware

ESP32 DevKit V1

4-channel relay module

ESP32 USB cable

## 5.2 V1 Hardware / Pending Delivery

INMP441 I2S microphone

MAX98357A I2S amplifier

4 ohm 3W speaker

5V 2A power adapter

Breadboard

Jumper wires

0.96 inch SSD1306 OLED

3-5 push buttons

Motors, movement and additional sensors are intentionally skipped for V1.

# 6. Robot Responsibilities

Wi-Fi connection

Device identity

Secure backend communication

Microphone input

Speaker output

OLED display

Buttons

Relay control

Health/heartbeat reporting

Local state management

Safe fallback behavior

Command acknowledgment

Backend responsibilities: authentication, AI, speech services where applicable, business database, business logic, permissions, automation, reporting and integrations.

# 7. ESP32 Hardware Pin Plan

The following is a provisional starting pin map. It must be verified against the actual hardware modules before final wiring and firmware lock.

| Component | ESP32 Pin |
| --- | --- |
| OLED SDA | GPIO 21 |
| OLED SCL | GPIO 22 |
| INMP441 BCLK | GPIO 27 |
| INMP441 WS/LRCLK | GPIO 14 |
| INMP441 DATA | GPIO 34 |
| MAX98357A BCLK | GPIO 27 |
| MAX98357A LRC | GPIO 14 |
| MAX98357A DATA | GPIO 13 |
| Relay 1 | GPIO 16 |
| Relay 2 | GPIO 17 |
| Relay 3 | GPIO 18 |
| Relay 4 | GPIO 19 |
| Listen button | GPIO 32 |
| Stop button | GPIO 33 |
| Confirm button | GPIO 25 |
| Menu button | GPIO 26 |

## Hardware rules

Do not assume the relay module input/power arrangement.

Verify whether the relay board is active-LOW or active-HIGH before final firmware.

Do not connect speaker terminals to GND when using a bridged MAX98357A output.

INMP441 logic power should use the appropriate 3.3V supply.

Final power wiring must prevent amplifier/relay load from brown-out resetting the ESP32.

Recheck pin conflicts when real modules are assembled.

# 8. Robot State Machine

BOOT -> WIFI_CONNECTING -> BACKEND_CONNECTING -> READY / IDLE

READY -> LISTENING -> PROCESSING -> EXECUTING -> SPEAKING -> READY

Any state -> ERROR -> recovery

Cloud failure -> OFFLINE_FALLBACK

| State | Purpose |
| --- | --- |
| BOOT | Initialize hardware and configuration. |
| WIFI_CONNECTING | Connect to configured Wi-Fi. |
| BACKEND_CONNECTING | Authenticate and register/heartbeat. |
| READY | Robot is ready for interaction. |
| LISTENING | Capture user speech. |
| PROCESSING | Wait for speech/AI result. |
| EXECUTING | Execute validated action. |
| SPEAKING | Play response audio. |
| ERROR | Display/report error and recover. |
| OFFLINE_FALLBACK | Limited local commands and recovery messaging. |

# 9. Voice Interaction

## 9.1 Primary Interaction

The robot is voice-first. Text chat is not a primary product feature.

## 9.2 Robot Name Activation

Normal background conversation
    -> Ignore

"ROBOT_NAME, turn on relay one."
    -> Wake
    -> Listen
    -> Process

## 9.3 Push-to-Talk

Press Listen
    |
    v
Listening
    |
    v
Capture command
    |
    v
Process

## 9.4 Languages

English

Hindi

Marathi

The app should allow selection of the robot speaking language.

# 10. Voice Pipeline

Microphone
   |
   v
Audio capture
   |
   v
Speech-to-text
   |
   v
User text
   |
   v
Intent / AI service
   |
   v
Structured action OR answer
   |
   v
Business/robot execution
   |
   v
Text response
   |
   v
Text-to-speech
   |
   v
Speaker

# 11. AI Architecture

## 11.1 Gemini's Role

Understand natural language.

Identify intent and extract entities.

Choose approved tools/actions.

Explain business information.

Generate conversational responses.

Use business context when answering.

Gemini must NOT directly execute SQL.

Gemini must NOT directly control GPIO.

Gemini must NOT write arbitrary files.

Gemini must NOT bypass permissions.

Gemini must NOT be the source of truth for stock/prices.

Gemini must NOT be treated as a security authority.

## 11.2 Structured Command Principle

User: "Turn on the first relay."

AI output:
{
  "type": "action",
  "action": "relay",
  "relay": 1,
  "state": "on"
}

Backend validates before execution.

# 12. Business AI Tool / Action Layer

get_business_summary

get_product_stock

add_inventory

reduce_inventory

create_bill

get_bill

get_customer

get_customer_balance

record_payment

create_reminder

get_reminders

get_reports

control_relay

get_robot_status

Every action needs a schema, permission requirements, validation, audit behavior, expected result and error behavior.

# 13. Business Instruction TXT File

The owner can upload a business instruction .txt file. It customizes AI context, business preferences and operational guidance.

Business Name: Example General Store
Business Type: Grocery

Working Hours: 8 AM - 9 PM

Minimum Sugar Stock: 20 kg
Minimum Rice Stock: 30 kg

Credit is allowed only for regular customers.

Payment reminders should be created after 3 days overdue.

Owner prefers Marathi voice responses.

The file is business context, not a security bypass. Backend permissions always win.

# 14. Inventory

## Core

Product list

Add product using barcode

Edit product

Add stock

Reduce stock

Current stock

Minimum stock

Low-stock alert

Stock history

Supplier information

Purchase history

Automatic reorder suggestion

## Future

Richer barcode/QR workflows

# 15. Billing

## Core

Create bill

Add products

Quantity

Price from database

Automatic total calculation

Generate invoice

Billing history

AI voice billing through robot

## Useful additions

GST/tax

Discount

Print

PDF

Digital invoice

Cancel/refund

# 16. Customer Management

## Core

Customer list

Add customer

Edit customer

Purchase history

Pending balance

Payment history

Top customers

AI customer insights

## Useful additions

Customer notes

Customer preferences

# 17. Payments

## Core

Record payment

Pending payments

Payment history

Outstanding balance

## Useful additions

Payment reminders

## Future

Online payments

Automatic payment reconciliation

# 18. Reminders

## Core

Create reminder

View reminders

Complete/delete reminder

Voice-created reminders

Due-task notifications

## Useful additions

Recurring reminders

AI suggested reminders

## Future

AI learning of repeated routines

# 19. Reports

## Core

Daily sales report

Weekly sales report

Inventory report

Pending payment report

AI-generated business summary

## Useful additions

Monthly sales

Profit

Product report

Customer report

Charts

AI recommendations

# 20. Automation Engine

LOW STOCK
  -> alert owner

SALE CREATED
  -> reduce inventory

BILL CREATED
  -> update customer history

PAYMENT CREATED
  -> update outstanding balance

Due reminder -> notification

Daily closing

Automatic purchase list

AI-created workflows

Custom IF/THEN automation

Automation must be deterministic where possible.

# 21. Proactive Manager Behavior

Low stock

Pending/overdue payment

Important reminder

Daily business summary

Notify when the event is meaningful and actionable; avoid notification spam.

# 22. Multi-Action Command Handling

User:
"Add 20 kg sugar and tell me what is now below minimum stock."

Workflow:
1. Find sugar
2. Validate unit
3. Add stock
4. Record inventory transaction
5. Recalculate stock state
6. Check minimum-stock rules
7. Return results

Multi-step workflows must be implemented as backend services, not free-form model behavior.

# 23. Mobile App

Technology: Flutter

Dashboard

Robot

Inventory

Billing

Customers

Payments

Reminders

Reports

WhatsApp

Automation

Settings

## 23.1 Dashboard

Today's sales

Orders/transactions

Pending payments

Low-stock alerts

Important notifications

Robot online/offline

## 23.2 Robot Screen

Online/offline

Microphone status

Speaker status

OLED state

Relay state

Diagnostics

Permitted remote controls

# 24. WhatsApp

Send invoice

Payment reminder

Order confirmation

Customer inquiry handling

AI-generated replies

Future: automated WhatsApp workflows and promotions. Use the official platform/API and follow its policies.

# 25. Authentication and Permissions

OWNER
  - full access

STAFF
  - limited permissions

Sensitive action
  -> authenticate
  -> authorize
  -> validate
  -> audit
  -> execute

# 26. Robot Security

Assign each robot a unique identity, e.g. ROBOT-001.

Map the robot to its business/tenant.

Authenticate robot requests.

Do not expose uncontrolled public GPIO endpoints.

Do not embed unrestricted Gemini/API secrets in firmware.

Do not trust arbitrary commands.

Do not let the model execute arbitrary SQL.

Do not allow client-side robot impersonation.

# 27. Backend API Principles

GET  /health

POST /auth/login

POST /robots/register
POST /robots/heartbeat
GET  /robots/{robot_id}/status

POST /robots/{robot_id}/commands
POST /robots/{robot_id}/commands/{command_id}/result

POST /ai/process

GET  /products
POST /products
PATCH /products/{id}

POST /inventory/transactions
GET  /inventory

POST /customers
GET  /customers
GET  /customers/{id}

POST /bills
GET  /bills
GET  /bills/{id}

POST /payments
GET  /payments

POST /reminders
GET  /reminders

GET  /reports/daily
GET  /reports/weekly
GET  /reports/monthly

POST /business/instructions
GET  /business/instructions

GET  /audit

These are logical contracts; exact naming can be normalized during implementation.

# 28. Robot Command Lifecycle

PENDING
  |
  v
VALIDATED
  |
  v
SENT
  |
  v
RECEIVED
  |
  v
EXECUTING
  |
  v
SUCCESS / FAILED

{
  "command_id": "cmd_123",
  "robot_id": "ROBOT-001",
  "action": "relay",
  "relay": 1,
  "state": "on"
}

Robot result:
{
  "command_id": "cmd_123",
  "status": "success",
  "message": "relay_1_on"
}

# 29. Heartbeat

{
  "robot_id": "ROBOT-001",
  "wifi": "connected",
  "backend": "connected",
  "mic": "ok",
  "speaker": "ok",
  "oled": "ok",
  "relay": "ok",
  "firmware": "0.1.0"
}

Backend stores last heartbeat, online state, firmware version and health status.

# 30. Offline / Fallback Mode

Basic approved local relay control

Display system state

Basic button functions

Show 'Cloud unavailable'

Retry connection

The robot must degrade gracefully rather than freeze when cloud AI/backend is unavailable.

# 31. Database Design

users
businesses
robots

products
inventory_transactions
suppliers

customers
bills
bill_items
payments

reminders
notifications

business_instructions

ai_activity
robot_commands
audit_logs

Every important business record should be associated with a business/tenant ID.

# 32. Multi-Tenant Design

business_id = BUSINESS_001
robot_id = ROBOT_001

Even the first demo should be designed so future multi-branch/multi-business support does not require a database redesign.

# 33. Audit Logging

User

Robot

Action

Timestamp

Source

Status

Relevant entity

Result/error

18:20:21
Owner
Robot ROBOT-001
ADD_INVENTORY
Sugar +20 kg
SUCCESS

# 34. Error Handling

UNKNOWN_PRODUCT

INSUFFICIENT_STOCK

CUSTOMER_NOT_FOUND

PERMISSION_DENIED

ROBOT_OFFLINE

AI_UNAVAILABLE

INVALID_COMMAND

DUPLICATE_COMMAND

NETWORK_ERROR

Errors must be logged, human-readable, recoverable where possible, and safe.

# 35. Development Repository Structure

business-ai-robot/
│
├── README.md
├── docs/
│   └── ANTIGRAVITY_SPEC.md
│
├── backend/
│   ├── app/
│   │   ├── main.py
│   │   ├── config.py
│   │   ├── api/
│   │   ├── models/
│   │   ├── schemas/
│   │   ├── services/
│   │   ├── security/
│   │   ├── automation/
│   │   └── ai/
│   ├── tests/
│   └── requirements.txt
│
├── robot/
│   ├── firmware/
│   │   ├── main/
│   │   ├── hardware/
│   │   ├── network/
│   │   ├── audio/
│   │   ├── display/
│   │   ├── commands/
│   │   └── config/
│   └── tests/
│
├── app/
│   └── flutter_app/
│
└── deployment/
    ├── Dockerfile
    └── cloud/

# 36. Development Order

| Phase | Deliverable |
| --- | --- |
| Phase 0 — Foundation | Repository, environment, configuration, Git, README. |
| Phase 1 — Backend | FastAPI, /health, configuration, structured logging. |
| Phase 2 — Database | Schema, migrations, core CRUD. |
| Phase 3 — Security | Authentication, authorization, robot identity, audit log. |
| Phase 4 — Robot API | Registration, heartbeat, command, result acknowledgment. |
| Phase 5 — ESP32 Base | Wi-Fi, backend connection, heartbeat, relay control. |
| Phase 6 — Virtual Voice | PC microphone, STT, AI processing, ESP32 command, PC TTS. |
| Phase 7 — Real Audio Hardware | INMP441 capture, MAX98357A output, speaker. |
| Phase 8 — Robot UI | OLED states and buttons. |
| Phase 9 — AI Context | Business instruction file, structured tools, business context. |
| Phase 10 — Inventory | Complete core inventory workflow. |
| Phase 11 — Billing | Complete core billing workflow. |
| Phase 12 — Customers | Records, history and balance. |
| Phase 13 — Payments | Payment records and balances. |
| Phase 14 — Reminders | Reminders and notifications. |
| Phase 15 — Reports | Daily/weekly summaries and reports. |
| Phase 16 — Automation | Deterministic workflows and proactive alerts. |
| Phase 17 — Flutter App | Build app on stable backend APIs. |
| Phase 18 — WhatsApp | Official API integration. |
| Phase 19 — Cloud Deployment | Deploy backend and database. |
| Phase 20 — Exhibition Hardening | Failure testing, security, power/restart, long-run testing. |

# 37. Current First Milestone

The immediate target is NOT inventory or billing. The first complete end-to-end feature is VOICE RELAY CONTROL.

Owner
  |
  v
"Robot, turn on relay one."
  |
  v
Speech-to-text
  |
  v
AI
  |
  v
{
  "action": "relay",
  "relay": 1,
  "state": "on"
}
  |
  v
Backend validation
  |
  v
Robot command
  |
  v
ESP32
  |
  v
Relay 1 ON
  |
  v
Confirmation

# 38. Acceptance Tests

## Robot

Boot

Wi-Fi

Backend registration

Heartbeat

Commands

Acknowledgments

Safe relay control

Recovery from network loss

## Voice

Push-to-talk

Robot-name activation

Background conversation ignored

English

Hindi

Marathi

TTS

Failure handling

## AI

Natural language

Structured output validation

Safe rejection

No direct DB access

Business context

Business TXT instructions

Permission/confirmation

## Business

Inventory updates

Database-driven billing

Customer balance

Payment balance

Reminders

Reports

Deterministic automation

## App

Login

Dashboard

Inventory

Billing

Customers

Payments

Reminders

Reports

Robot status

Settings

## Exhibition

PC not required

Robot works over Wi-Fi

Phone uses same backend

Backend online

Authenticated APIs

Fallback works

Demo repeats reliably

# 39. Non-Goals / Things Not to Build Yet

Motors

Movement

Complex sensors

Computer vision

Autonomous navigation

Multi-branch complexity

Custom automation builder

Advanced routine learning

Online payment reconciliation

Firmware OTA

Unnecessary UI animations

Priority is reliable business workflow plus reliable robot interaction.

# 40. Design Principles

Database is the source of truth.

Backend is the authority.

AI is a reasoning/interface layer, not unrestricted system authority.

ESP32 is the physical controller.

App and robot share the same backend/database.

Sensitive actions need permissions.

Every important action is auditable.

Hardware failures must be recoverable.

Cloud failure must not crash the robot.

Build and test one vertical workflow at a time.

# 41. Antigravity Agent Instructions

Read this specification before changing architecture.

Do not create fake integrations when a real interface is available.

Do not hard-code secrets into source code.

Do not bypass backend validation.

Do not allow Gemini to directly execute arbitrary system/database operations.

Preserve modular interfaces.

Write tests with each meaningful backend feature.

Build one working vertical slice before moving to the next.

Keep robot firmware non-blocking where practical.

Report implementation status clearly.

Do not silently change approved product features.

When a hardware detail is uncertain, mark it provisional instead of inventing certainty.

Keep development and production configurations separate.

Do not make the PC a hidden production dependency.

Keep the system deployable to cloud infrastructure.

Prefer simple, reliable implementation over unnecessary complexity.

Every business write operation must be auditable.

Every robot command needs an ID and result.

Every major failure should have a recovery path.

The final acceptance test is Robot + Phone + Internet, no PC.

# 42. Final Architecture Lock

| Layer | Technology / Component |
| --- | --- |
| Robot | ESP32 DevKit V1 |
| Audio input | INMP441 |
| Audio output | MAX98357A + 4 ohm 3W speaker |
| UI | SSD1306 OLED + buttons |
| Actuation | 4-channel relay |
| Backend | Python + FastAPI |
| AI | Gemini API |
| Mobile app | Flutter |
| Database | PostgreSQL or approved equivalent |
| Authentication | Secure user + robot authentication |
| Transport | HTTPS initially |
| Production deployment | Google Cloud Run |
| Development | PC may temporarily provide microphone/speaker and local backend |
| Exhibition | Robot + Phone + Cloud backend + Internet, no PC |

# 43. Exhibition Success Definition

At the exhibition, a visitor should be able to see that the robot is not merely a chatbot, inventory app, ESP32 relay project, or voice assistant. It is a connected AI business operations system with a physical robot interface.

🤖 ROBOT
   |
"Robot, add 20 kg sugar."
   |
   v
AI
   |
   v
DATABASE
   |
   +----> Inventory
   |
   +----> Mobile App
   |
   v
Robot:
"20 kg sugar added."

Then:
"Robot, how much sugar is left?"
-> Live database answer

Then:
"Turn on relay one."
-> Real physical relay response